| Name: ___________________________________ | Date: ______________ |
| --- | --- |

PT Course Day 3 Quiz

|  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |
|  |  |  |  |  |  |  |  |  | 6 |  |  |  |  |  |
|  |  |  |  | 7 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |

| Across 2. During a Visible Inspection, The light intensity is required to be 100 _______ - _________ at the surface being inspected. 4. ______________ penetrant inspection kits are for penetrant inspection of parts too large to be brought into the inspection lab, or for laboratories which process only a minimum number of parts requiring penetrant inspection. 5. Black light intensity will also be affected by voltage variations. A bulb that produces acceptable intensity at 120 volts will produce significantly _____ at 110 volts. 7. Black lights should never be used with a ___________ filter as output of white light and harmful black light will be increased. 9. Light intensity measurements are made using a ________________. 10. Visible-dye or color-contrast penetrants contain a _______ dye dissolved in the penetrating oil | Down 1. _____________ action is defined as the tendency for a liquid to penetrate or migrate into small openings, such as cracks, pits, or fissures. 3. Surface tension and __________ action are only two requirements of a penetrant. 6. During fluorescent Penetrant inspection, the source of ultraviolet light is often a __________ arc lamp with a filter 8. When using a visible penetrant, the intensity of the ________ light is of principal importance. |
| --- | --- |